TECHNICAL AGREEMENT

技术规格书

| 技术规格书编号 Project 项目 Description 描述 Model 型号 Qty. 数量 Classification 船级社 Owner 船东 Purchase 采购方 Designer 设计方 Manufacturer 制造商 Supplier 供货方 Signed at 签字地点 | : : : : : : : : : : : | Marine Propulsion Engine 船用推进发动机 K38-M (1007kw/1900rpm） 3 SETS PER SHIP 3套/船 BV Octagen 广州赢亮设计研发有限公司 MS 重庆康明斯发动机有限公司 Fujian MT Power Co.,Ltd 福建明泰船舶科技有限公司 |
| --- | --- | --- |

Section A－TECHNICAL SPECIFICATION OF MARINE PROPULSION ENGINE

A部分－船用推进发动机的技术规格

| Engine Model 发动机型号 Performance of Engine: 发动机的性能参数： Power: 发动机功率： - Rated Speed额定转速 Rating Conditions: 功率的标定工况： -Air Pressure 大气压 -Air Temperature 进气温度 -Sea Water Temperature 海水温度 -Relative Humidity 相对湿度 -Power Deration 功率修正 Rating definition 功率定义 Dimensions 尺寸 Dry Weight 干重 General Configuration 结构综述 -No. of cycle 冲程数 -No. of Cylinders 气缸数 -Bore x Stroke 缸径 x 冲程 - Compression Ratio 压缩比 -Displacement 排量 -Configuration 机型结构 -Aspiration 吸气方式 -Revolution Direction 旋转方向 Fuel System 燃油系统 -Maximum Allowed Restriction to Fuel Pump 燃油泵前最大允许的进油阻力 -Maximum Allowed Return Line Pressure 最大允许的回油阻力 -Fuel Consumption at rated power 额定点的油耗 Fuel Consumption is based on fuel of 35 °C API gravity at 16 °C (60 °F) having LHV of 42, 780 kj/kg (18, 390 Btu/Ib) and weighing 838.9 g/liter (7.001 Ib/U.S. gal). Exhaust System 排气系统 -Maximum Allowed Back Pressure 最大允许的排气阻力 -Recommended New Engine Allowed Back Pressure 推荐新机允许的排气阻力 Air Induction System 进气系统 -Maximum Allowed Intake Restriction 最大允许的进气阻力 Lube Oil system 润滑油系统 -Oil Pan Capacity, High/Low (Standard) 油底壳的容量, 高位/低位(标准) -Total Lube System Capacity-Sumo plus Filters 标准油底壳时的润滑油容量 -Maximum Operational Angularity of oil pan （Intermittent） 油底壳的倾斜角（瞬时） Front Down 前下倾 Front Up 前上倾 Side to Side 左右摇动 Electrical System 电气系统 Starting system 启动系统 Drive system 驱动系统 Operation Fluids & Lubricants 燃油和润滑油的规格 -Fuel Oil 燃油 -Lube Oil 润滑油 Coolant 冷却淡水 Manufacturer 制造商 | : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : : | K38-M，重庆康明斯（CCEC） （每船三台，中间一台需前端带消防泵） （左右两台驱动齿轮箱PTO/PTI 带动轴发，属于齿轮箱包） 1007kW(1350BHP) 1900 rpm In accordance with ISO-8665 reference conditions: 符合 ISO-8665 的标准 100 kPa 0~~50°C ≤36℃ ≤95% No power correction required when operating at 45°C ambient temperature. When ambient temperature exceeds 45°C, power is derated by 2% per11°C increase. 发动机可以在环境温度至45 ℃时没有功率修正；当温度再上升，每上升 11℃ , 功率下降 2％。 Heavy Duty 重载功率（HD） 用于在每12小时满功率运行不超过10小时的变功率持续应用,降功率使用时应等于或小于额定功率的80%.这是lS015550燃油截止功率等级。典型的应用包括: 排水型船如拖网渔船，围网渔船，经常减速或转速负荷恒定的拖船，高速船只的应用如渡轮和船员交通艇。典型的辅机应用包括:货船泵,动态定位用的推进器。典型的功率因数=60-75%，年操作时间不超过6000小时。 ~2456mm×1462mm×2083mm（长×宽×高mm） 认可图确认最终尺寸 ～4538kg 4 12 12缸 159mm x 159mm 13.9:1 38Liters 38升 Cylinder In V 气缸V型排列 Turbocharged & LTA 增压、低温中冷 Counterclockwise viewed to flywheel 面向飞轮, 飞轮逆时针运转 （柴油机固定转向，齿轮箱根据螺旋桨区分左右机） （The rotation of Engine is fixed. The left and right gearbox is according to the propeller.） 14 kPa (Clean Filter干净滤清器) 27 kPa (Dirty Filter 脏滤清器) 22 kPa 260.5L/h+5% 油耗基于35 °C API, 16 °C (60 °F) 时的热值 LHV 42, 780 kj/kg (18, 390 Btu/Ib) 和比重838.9 g/liter 10 kPa 5 kPa 4 kPa (Clean Filter干净滤清器) 6 kPa (Dirty Filter 脏滤清器) 128.5 /85.7L 135.1L 30°C 30°C ±45°C 24 V DC 直流 24 伏 DC 24V Starter 电启动马达 SAE 0 flywheel housing with 18in in clutch adaptation SAE 0号飞轮壳与18in离合器相配 Diesel Oil is Grade No. 2-D requirements of the American Society for Testing Or China Fuel Type 0 – GB 252-87 (Grade 1 & Special Grade ) as equivalent, or China Fuel Type -10 – GB 252-87( grade 1 & special grade ) as equivalent 美国标准2号柴油 中国标准 0 号(国标252-87), 或－10 号(国标252-87) SAE 15W-40, Grade CF-4/SG, CG-4/SH or CH-4/SJ 标准SAE 15W-40等级CF-4/SG, CG-4/SH 或CH-4/SJ的润滑油，推荐选用康明斯专用机油。 50-50 Ethylene Glycol/Water and Adequate concentration of DCA4 50％的软水和50％的乙烯已二醇或丙烯已二醇所混合而组成的防冻液, 并添加适量的DCA4溶剂。推荐选用康明斯专用防冻液 Chongqing Cummins Engine Co, .Ltd 重庆康明斯发动机有限公司 |
| --- | --- | --- |

Section B SUPPLY SCOPE

B部分供货范围

- Engine K38-M,  with Following Equipment:

发动机K38-M，配备如下的附件

  - Air System  空气系统

    - Air Cleaner 空气滤器

    - Turbocharger  增压器

    - After-cooler 中冷器

    - Inlet Restriction Indicator 进气阻力指示器

    - Crankcase Vent Line 曲轴箱透气软管

  - Cooling System 冷却系统

    - Fresh Water Pump and淡水泵

    - Thermostat , For Fresh Water 冷却水节温器

    - Water filter 水滤器

    - HEAT EXCHANGER   热交换器（板冷）

    - EXPANSION TANK   膨胀水箱

    - RAW WATER PUMP   机带海水泵

    - FLEXIBLE SECTION TUBE 配海水冷却水进出口挠性接头，散件供应，带船级社型式认可

    - 1.2.8 配淡水高低温管挠性软管，散件供应

  - Lub Oil System 滑油系统

    - Oil Pump 机油泵

    - Oil Filter 滑油滤器

    - Oil Cooler滑油冷却器

    - Oil Pan 油底壳  （润滑方式是闭式循环，湿式油底壳）

    - Oil Dipstick 机油尺

    - Oil Scavenge Hand Pump With Outlet Flexible Line 手动污油泵及连接油底壳软管，带船级社型式认可

  - Fuel Oil System  燃油系统

    - FUEL FILTER 燃油滤器

    - FUEL PUMP WITH MECHNICAL GOVERNOR 机械调速的油泵（加装电子调速）

    - Fuel Float Tank  (Loose Supply)   浮子油箱 散供 （燃油日用柜液位高于曲轴中心线1.6米时才需要）

    - (Fuel Float Tank only required when the return line is more than 1.6 meters above crankshaft centerline)

    - Water Separator   (Loose Supply)  双联油水分离器（国产） (散供）

    - Fuel inlet/outlet flexible section BV type approval 燃油进出口软管带船级社型式认可

    - 注：无燃油泄漏报警，该PT燃油系统属于低压系统，无双层高压油管及燃油泄漏，符合船级社规范！

- Note: There is no fuel leakage alarm. The PT fuel system belongs to the low-pressure system, without double layer high-pressure fuel pipes and fuel leakage, which complies with the classification society's regulations!

  - Exhaust System 排气系统

    - Exhaust Elbow 排气弯管

    - Exhaust Manifold With Heat Shield排气歧管带隔热罩

    - Exhaust Flexible Section With Counter Flange (Loose Supply) 排气波纹管带配对法兰（散供）

    - Flanges include gaskets, mounting bolts, and nuts，as loose （法兰包括垫片、安装螺栓、螺母） 散供

    - Exhaust Y pipe two-in-one  排气二合一 汇合口

  - Starting and driver System 启动系统

    - DC 24V Starter  电启动马达

    - Charger  机带充电机

  - Driver system 驱动系统

    - Flywheel and  Housing  SAE 18”飞轮  SAE 0# 罩壳 ,

    - Vulkan Coupling for Gear Box ，with bolts 伏尔康高弹18” 用于驱动齿轮箱 ，含高弹飞轮安装螺栓

    - One Engine equipped with a PTO shaft at the front end for drive the FiFi Pump 其中一台前端带PTO轴，用于驱动消防泵

2. Control monitoring system控制监测系统

2.1. Control panel with parameters as following机旁控制箱具有如下的监控和显示功能：

2.1.1显示如下参数：

- Oil Press Gauge 油压表

- Water Temp. Gauge  水温表

- Tachometer/ Hour-meter  转速表/小时计

- Oil Temperature Gauge 油温表

- Exhaust Temperature Gauge 排温表

- Voltage Meter  电源电压表

2.1.2 Switch and button 开关和按钮

    - Power Switch电源开关

    - Start Button 起动按钮

    - Stop Button 停车按钮

    - Reset And Bell Stop  Button 复位和消音按钮

    - Lamp Test 试灯按钮

    - Emergency Stop Button，With Protection Cover  紧急停车带保护罩

    - Alarm buzzer 蜂鸣器

    - Local / Remote Select Switch机旁/遥控控制切换开关

2.1.3 Alarm List View  报警显示

    - Engine Lube Oil Pressure Low Alarm滑油压力低报警

    - Engine Lube Oil Temperature High Alarm滑油温度高报警

    - Engine Coolant Temperature High Alarm冷却水温度高报警

    - Engine Over-speed Alarm And Shutdown发动机超速报警和停车

    - Exhaust Temperature High Alarm 排气温度高报警

    - Engine Lube Oil Pressure Low-Low Shutdown  滑油压力过低停机

    - Engine Coolant Temperature High-High Shutdown  冷却水温度过高停机

    - Start Battery Volt Low Alarm 启动电瓶电压低报警

2.1.4 Customer Interface提供下列用户接口（以接口会议为准）

    - Engine Lube Oil Pressure Low Alarm 滑油压力低报警

    - Engine Lube Oil Temperature High Alarm 滑油温度高报警

    - Engine Coolant Temperature High Alarm 冷却液温度高报警

    - Engine Over Speed Shutdown 发动机超速停车

    - Exhaust Temp  High Alarm 排气温度高报警

    - Engine Lube Oil Pressure Low-Low Shutdown  滑油压力过低停机

    - Engine Coolant Temperature High-High Shutdown  冷却水温度过高停机

    - Running  运行

    - Common alarm 公共报警

    - 4-20mA Engine Speed Control Input  4-20mA发动机转速控制输入

    - 4-20mA Engine Actual Speed Control 4-20mA发动机实际转速反馈输出

2.1.5  Remote  panel, with cables, Loose supply  远程仪表板，带线，散供

- Drawings and  Document 图纸和文件

  - 4 copies of the followings approval drawings & documents should be submitted within 4 weeks after signing the contract 在合同签订后 4周内将提供 如下认可图纸

    - Installation Drawings 安装图

    - System Diagram 系统图

    - Data Curve 特性曲线

    - Technical sheet 技术数据

    - Accessories drawing 附件图纸

    - Torsional Vibration Calculation Parameters扭振计算参数

    - Electrical Principle Diagram And External Wiring Diagram电气原理图和外部接线图

  - One copy of final document  should be submitted while engine delivered发动机提交时提1份完工文件

    - Parts Catalog 零件图册（电子版）

    - Installation Direction 安装指南（电子版）

    - Operation and Maintenance Manual 操作保养手册（电子版）

    - Engine Test Report  发动机测试报告

    - BV  production certificate BV产品证书

    - Specialist tools 专用工具清单

    - Emissions Certificate IMO II  发动机IMO II排放证书及氮氧化物手册

    - TVC  扭振计算

- Provide 3 working days per ship commissioning in shipyard without additional charge.

免费提供每船7个工作日的船厂调试。

- One Set Of Tools  Made In China康明斯标准工具一套,  中国造(Attached 1)

- After delivery，cummins engine in accorda版 with the regulation on the warranty guarantee 发动机交付后按康明斯保修条例进行保修

- If there are differences in above，the Chinese version will prevail 以上中英文如有差异，以中文为准。

- We will discuss and solve friendly about the other problems not mentioned in this agreement 其他未尽事宜，双方友好协商解决。

Attached List  1                 CUMMINS TOOLS SET

附表1：                           康明斯工具清单

| No. | Name | Spec./Quantity |
| --- | --- | --- |
| 序号 | 名称 | 规格/数量 |
| 1 | Hinge Handle: 活络扳手 | 1010034， 1 Pc.共1件 |
| 2 | Filter disassembling tool 滤清器拆装工具 | 4914504-20， 1 Pc.共1件 |
| 3 | Elbow extension : 弯头接杆 | MH713817， 1 Pc.共1件 |
| 4 | Open box wrench :3/4” 两用扳手 3/4” | 1010006， 1 Pc.共1件 |
| 5 | 1/2”Sockets(3/4”): 1/2”套筒头（3/4”) | 1010077, 1 Pc.共1件 |
| 6 | 1/2”Sockets(15/16”): 1/2”套筒头（15/16”) | 1010080, 1 Pc.共1件 |
| 7 | 1/2”Sockets(1-1/4”): 1/2”套筒头(1-1/4”) | 1010084, 1 Pc.共1件 |
| 8 | Socket arbors: 变方接头1/2”*3/8”*1” | 1010099， 1 Pc.共1件 |